You will find below the link and the instructions to download and install the Global D libraries. For more information, visit our website https://www.globald.com/en/implantology/implantology-digital-solutions/cad/

| 3shape |  |
| --- | --- |
| 3Shape folder download link containing the libraries: Global_D_3shape_library_complete_V2.1.1c+In-koneWD_V2.4.4_Pack.zip - Link: https://monextra.net/wl/?id=PN5sJaGTm89nET00dCrEzo0AXD5HTFEn - Password: Gld2018-3shape - (Contains dme files, multi-language documentations and installation instructions) Unzip the following files: Global_D_3Shape_library_Complete_V2.1.1c_2019-01-24.dme Global_D_3Shape_library_WD_V2.4.4_20211129.dme First install : Run “Dental System Control Panel”, Go to “Tools” and select “Import/Export” Click to “import materials” Select the file “Global_D_3Shape_library_Complete_V2.1.1c_2019-01-24.dme” Click to “import materials” Select the file “Global_D_3Shape_library_WD_V2.4.4_20211129.dme” Uncheck the “materials” box (the materials have already been imported before) Return to the home page Go to “Site Setting” and select “Manufacturing processes” Select (left table) "Hard machining" and/or "Milling R0.4" and/or "Milling R0.5" if they exist, or the usual process that you select for metal machining Associate the manufacturer (you in right table) with the Global D Materials (center table) by checking “Global D Ti Premilled”, “Global D CoCr Compliance” and “Global D Ti Compliance” Exit the System Control Panel and “save changes before closing” Added in-Kone WD librarie: Run “Dental System Control Panel” Go to “Abutments” and select “implant system” Check Global D library versions (in the list starting from the end) The version must be 2.1.1c or higher (otherwise follow “Update of 3shape library” instructions) Go to “Tools” and select “Import/Export” Click to “import materials” Select the file “Global_D_3Shape_library_WD_V2.4.4_20211129.dme” Uncheck the “materials” box (the materials have already been imported before) Do not update, select “keep existing” Return to the home page Go to “Site Setting” and select “Manufacturing processes” Select (left table) "Hard machining" and/or "Milling R0.4" and/or "Milling R0.5" if they exist, or the usual process that you select for metal machining Associate the manufacturer (you in right table) with the Global D Materials (center table) by checking “Global D Ti Premilled”, “Global D CoCr Compliance” and “Global D Ti Compliance” Exit the System Control Panel and “save changes before closing” Update of 3Shape library: Run Dental “System Control Panel” Go to “Abutments” and select “implant system” Check Global D library versions (in the list starting from the end) Delete all Global D implant system, version greater to 2.1.1c Return to the home page Go to “Basic Elements” and select “Materials”, Select and delete in the left table all the materials starting with Global D…. Save (disk in header menu), Return to the home page Go to “Tools” and select “Import/Export” Click to “import materials” Select the file “Global_D_3Shape_library_Complete_V2.1.1c_2019-01-24.dme” Do not update, select “keep existing” Click to “import materials” Select the file “Global_D_3Shape_library_WD_V2.4.4_20211129.dme” Uncheck the “materials” box (the materials have already been imported before) Do not update, select “keep existing” Return to the home page Go to “Site Setting” and select “Manufacturing processes” Select (left table) "Hard machining" and/or "Milling R0.4" and/or "Milling R0.5" if they exist, or the usual process that you select for metal machining Associate the manufacturer (you in right table) with the Global D Materials (center table) by checking “Global D Ti Premilled”, “Global D CoCr Compliance” and “Global D Ti Compliance” Exit the System Control Panel and “save changes before closing” Selection menus: “Global D” (may be delete) “Global D V2” contains V2.1.1c library to use "Global D WD" additional library with only the prosthetic connection on In-Kone WD implant - Warning - The "GLOBAL D PC4.3 Implant studio" implant kit is only reserved for Implant Studio planning. |  |
| DentalWings |
| DentalWings folder containing the download link libraries: - Global_D_DW_library_complète_v2.2.2+In-Kone_WD_v2.4.4_Pack Link : https://monextra.net/wl/?id=fUMfHA8UaKIztSL6RJU6L9C7H28CXmeY - password: Gld2018-Dwos - Unzip Global_D_DW_library_complète_v2.2.2+In-Kone_WD_v2.4.4_Pack.zip - Extract the following files: - Global D Titanium Premilled S3DEL.xml - Global D v2.2.2.zip - Global D WD V2.4.zip If you cannot control the installation of a new library, contact your software vendor. Installation of the Dental Wings library: Select the icon parameter (gear) / Materials If “Global D” does not exist, create the manufacturer "Global D" Clicking the + sign at the bottom right of the window DWOS, Install or update Global_D_Titanium_premilled_S3DEL.xml material (right-click Global D / import material) Select the setting icon (gear) / Implant Kits Delete all manufacturers starting with Global D (select the row, right click "delete the manufacturer") Click on the + sign at the bottom right of the DWOS window Select import manufacturer implant kits Choose the file "Global D v2.2.2.zip" Click on the + sign at the bottom right of the DWOS window Select import manufacturer implant kits Choose the file "Global D WD V2.4.zip" Added in-Kone WD librarie: Select the icon parameter (gear) / Materials If “Global D” does not exist, create the manufacturer "Global D" Clicking the + sign at the bottom right of the window DWOS, Install or update Global_D_Titanium_premilled_S3DEL.xml material (right-click Global D / import material) Select the setting icon (gear) / Implant Kits Click on the + sign at the bottom right of the DWOS window Select import manufacturer implant kits Choose the file "Global D WD V2.4.zip" Selection menus: “Global D V2.2” contains V2.2.2 standard library to use "Global D WD V2.4" additional library with only the prosthetic connection on In-Kone WD implant - Warning - The implant kits "In-Kone + PC4.3" and "EVL + MULTI" are only available for coDiagnostiX implant plans via the Synergy module. |
| Exocad |
| Library download link from the exocad website: - Link : https://exocad.com/integration/dentalcad-library-integration#/all/tag-g - Select packages (+ Add to download list) GlobalD MultiUnit exocad libraries (10 libraries) GlobalD IL exocad libraries (2 libraries) GlobalD Lab Analog exocad libraries (1 library) - These libraries are validated by exocad, it’s possible to maintain the old versions V2.2e & V2.4.4 (Global D V2) but it is preferable to delete them. or Library download link from Global D Link : https://monextra.net/wl/?id=8BMhxEFI4rggBqCbuKBk5jixGjZOMxDo Mot de passe : Gld2018-Exocad Libraries & documentation Find the installation folders location: The 2 locations to find are: …\DentalCADApp\library\implant\ for single or plural abutment libraries, …\DentalCADApp\library\modelcreator\implant\ for the model library. The link to the DentalCADApp folder location can be identified by right-clicking on the Exocad executable icon (cad), then go to Properties and noting the path in the "Target" box, surch as “C:\Exocad\exocad-DentalCAD….\DentalCADApp\...”. This location may be different with some integrators, for example: Amann Girrbach : …\Ceramill software\CAD\library\, Zfx : …\Zfx Dental Design\ZfxCAD\library\ Exocad libraries installation: do not change folders names Unzip all files Go to …\DentalCADApp\librairy\implant\ Delete older Global D libraries if necessary (optional), Drop folders For CAD; For Exoplan. Global-D_EVL_conid; Global-D_twinKon_conid_multiunit. Global-D_In-Kone_conid; Global-D_EVL4_conid_multiunit. Global-D_MUA_conid_multiunit_support; Global-D_EVL5_conid_multiunit. Global-D_MULTI_conid_multiunit_support; Global-D_EVL33_conid_multiunit. Global-D_PAOF_conid_multiunit_support; Global-D_In-Kone_ST_conid_multiunit. Global-D_PC43_conid_multiunit_support; Global-D_In-Kone_WD_conid_multiunit. Go to …\DentalCADApp\librairy\modelcreator\implant Delete older Global D libraries, if necessary (optional), Déposer le dossier "Global-D_ModelCreator" All folders must contain files (.fda / .stl / .xml / .txt) and no folder. If there is one or more folders. - Selection Menus “Global-D - xxxx” New libraries (V3) to use “Global D V2” if maintain, old complete V2.2e library "Global D WD" if maintain, add-on with only the prosthetic connection on In-Kone WD implant |

The CAD-CAM connections provided:

In-Kone ST on implant and indexed Ti Base In-Kone Ø3.8 Gingival heights 0.6, 2, 3 and 4 mm or Ø5.5 heights 1.5, 2.2, 3 et 4 mm,

In-Kone WD on implant and indexed Ti Base In-Kone for healing screws Ø6.5, Gingival heights 1.5, 2.2, 3 et 4 mm,

EVL on implant and indexed Ti Base EVL (except EVL C),

EVL NI on implant and Ti Base non-indexed EVL (except Exocad – 100% panthera)

PC 4.3 on new conical abutments 0 ° and rights associated sockets, non-indexed (In-Kone, TwinKon)

PAOF on angulated conical abutments and associated sockets, non-indexed In-Kone (In-Kone, compatible with Hexacolor Ø4.7 angulated conical abutment)

MULTI on MULTI abutment and bases, implant EVLC, non-indexed (EVL, CLINIC, compatible with In-kone PC4.0 and Hexacolor conical abutment)

MUA on pillars and sockets MULTI angulated, non-indexed (EVL, CLINIC,).

In-Kone+PC4.3 or PC4.3 implant Studio reserved in synergy with a guided surgery planning,

EVL + MULTI  reserved in synergy with a guided surgery planning.

This library includes several heights of In-Kone bases and offers the possibility to the lab to shorten them to 3 mm (i.e. at the level of the middle groove). Warning 3D direct connections implants or abutments are false. Only the Panthera Dental Company is authorized to machine these connections. Machining Laboratory is possible via Tibase, premilled or compliance system.

We count on you to let us know your suggestions for correction or improvement.

Best regards,